- Addressing modes in 8085 microprocessor

Prerequisite – Addressing modes
The way of specifying data to be operated by an instruction is called addressing mode.

Types of addressing modes –
In 8085 microprocessor there are 5 types of addressing modes:

- Immediate Addressing Mode –
In immediate addressing mode the source operand is always data. If the data is 8-bit, then the instruction will be of 2 bytes, if the data is of 16-bit then the instruction will be of 3 bytes.

Examples:
MVI B 45 (move the data 45H immediately to register B)
LXI H 3050 (load the H-L pair with the operand 3050H immediately)
JMP address (jump to the operand address immediately)

- Register Addressing Mode –
In register addressing mode, the data to be operated is available inside the register(s) and register(s) is(are) operands. Therefore the operation is performed within various registers of the microprocessor.

Examples:
MOV A, B (move the contents of register B to register A)
ADD B (add contents of registers A and B and store the result in register A)
INR A (increment the contents of register A by one)

- Direct Addressing Mode –
In direct addressing mode, the data to be operated is available inside a memory location and that memory location is directly specified as an operand. The operand is directly available in the instruction itself.

Examples:
LDA 2050 (load the contents of memory location into accumulator A)
LHLD address (load contents of 16-bit memory location into H-L register pair)
IN 35 (read the data from port whose address is 01)

- Register Indirect Addressing Mode –
In register indirect addressing mode, the data to be operated is available inside a memory location and that memory location is indirectly specified by a register pair.

Examples:
MOV A, M (move the contents of the memory location pointed by the H-L pair to the accumulator)
LDAX B (move contains of B-C register to the accumulator)
LXIH 9570 (load immediate the H-L pair with the address of the location 9570)

- Implied/Implicit Addressing Mode –
In implied/implicit addressing mode the operand is hidden and the data to be operated is available in the instruction itself.

Examples:
CMA (finds and stores the 1’s complement of the contains of accumulator A in A)
RRC (rotate accumulator A right by one bit)
RLC (rotate accumulator A left by one bit)

## INSTRUCTION FORMAT

An instruction (instruction format) is a command to the microprocessor to perform a given task on a particular data. Each instruction (instruction format) is of two parts. One is task to be performed, called the operation code or opcode and the second one is the data to be operated on, called the operand. The operands or data can be specified in different ways. It may include an 8-bit or 16-bit data, an internal register. a memory location, or 8-bit or 16-bit address. In some instructions, the operand is implicit.

### Instruction Word Size

The 8085 instruction set is of three groups according to word size:

- One-word or one-byte instructions.

- Two-word or two-byte instructions.

- Three-word or three-byte instructions.

In the 8085 microprocessor, byte and words are synonymous because it is an 8-bit microprocessor. But, instructions are commonly referred to in terms of bytes rather than words.

#### One-byte instructions

A one-byte instruction includes a opcode and a operand in the same byte. Operand(s) are internal registers and are in the instruction in form of codes. If there is no numeral present in the instruction then that instruction will be of one-byte, for example, MOV C, A, RAL, and ADD B, etc. Table M.1 shows examples of one-byte instruction.

| Task | Opcode | Operand | Binary Code | Hex code |
| --- | --- | --- | --- | --- |
| Copy the content of accumulator in the register C. | MOV | C, A | 0100 1111 | 4FH |
| Add the contents of register B to the contents of the accumulator. | ADD | B | 1000 0000 | 80H |
| Invert each bit in the accumulator. | CMA | None | 0010 | 2FH |

Table M.1 shows the example of one-byte instruction

These instructions are of one-byte performing three different tasks. In the first instruction, operand and registers are specified. In the second instruction, the operand B is specific and the accumulator is not there. Similarly, in the third instruction, the accumulator is assume to be the implicit operand. These instructions are in 8-bit binary format in the memory and each requires one memory location.

#### Two-byte instructions

In a two-byte instruction, the first byte specifies the operation code and second byte specifies the operand. Source operand is a data byte and immediately following the opcode. If an 8-bit numeral is present in the instruction then that instruction will be of two-byte. Here, the numeral may be a data or an address. For example, in MVI A, 35H and IN 29H, etc. In a two-byte instruction, the first byte will be the opcode and the second byte will be for the numeral present in the instruction.

| Task | Opcode | Operand | Binary code | Hex code |
| --- | --- | --- | --- | --- |
| Load an 8-bit data byte in the accumulator. | MVI | A 35H | 0011 1110 0011 0101 | 3EH First byte 35H Second byte |

Table M.2 shows the example of two-byte instruction

#### Three-byte instructions

In a three-byte instruction, the first byte specifies the opcode, and the following two bytes specify the 16-bit operand. The second byte is the low-order operand and the third byte is the high-order operand. If a 16-bit numeral is present in the instruction then that instruction will be of three-byte. Here, the numeral may be a data or an address, for example, in LXI H,3500H and STA 2500H, etc.

| Task | Opcode | Operand | Binary code | Hex code |
| --- | --- | --- | --- | --- |
| Transfer the program sequence to the memory location 2085h | JMP | 2550H | 1100 0011 0101 0000 0010 0101 | C3 First byte 50 Second byte 25 Third byte |

Table M.3 shows the example of three-byte instruction

### Opcode Format

In the case of microprocessor, the instruction or operation are specified by using specific bit pattern unique for each instruction. These bit patterns contain all the information about operation, register used, memory to. The register and register pair are specified by using certain combination of bits. These combinations of bits are in table below.

| Register | Code |
| --- | --- |
| B | 000 |
| C | 001 |
| D | 010 |
| E | 011 |
| H | 100 |
| L | 101 |
| M | 110 |
| A | 111 |

Table M.4 shows Bit combination of Registers

| Register pair | Code |
| --- | --- |
| BC | 00 |
| DE | 001 |
| HL | 10 |
| AF or SP | 11 |

Table M.5 shows Bit combination of Registers

- Difference between CALL and JUMP instructions

CALL instruction is used to call a subroutine. Subroutines are often used to perform tasks that need to be performed frequently. The JMP instruction is used to cause the PLC (Programmable Logic Control) to skip over rungs.

The differences Between CALL and JUMP instructions are:

| SERIAL NO. | JUMP | CALL |
| --- | --- | --- |
| 1. | Program control is transferred to a memory location which is in the main program | Program Control is transferred to a memory location which is not a part of main program |
| 2. | Immediate Addressing Mode | Immediate Addressing Mode + Register Indirect Addressing Mode |
| 3. | Initialisation of SP(Stack Pointer) is not mandatory | Initialisation of SP(Stack Pointer) is mandatory |
| 4. | Value of Program Counter(PC) is not transferred to stack | Value of Program Counter(PC) is transferred to stack |
| 5. | After JUMP, there is no return instruction | After CALL, there is a return instruction |
| 6. | Value of SP does not changes | Value of SP is decremented by 2 |
| 7. | 10 T states are required to execute this instruction | 18 T states are required to execute this instruction |
| 8. | 3 Machine cycles are required to execute this instruction | 5 Machine cycles are required to execute this instruction |

Difference between SIM and RIM instructions in 8085 microprocessor:

| SR. NO. | SIM INSTRUCTION | RIM INSTRUCTION |
| --- | --- | --- |
| 1 | SIM stands for Set Interrupt Mask. | RIM stands for Read Interrupt Mask. |
| 2 | It is responsible for masking/unmasking of RST 7.5, RST 6.5 and RST 5.5. | It checks whether RST 7.5, RST 6.5, RST 5.5 are masked or not. |
| 3 | It resets to 0 RST 7.5 flip flop. | It checks whether interrupts are enabled or not and to check whether RST 7.5, RST 6.5 or RST 5.5 interrupts are pending or not. |
| 4 | The content of the Accumulator decides the action to be taken. So before executing the SIM instruction, it is mandatory to initialize Accumulator with the required value. | The contents of the Accumulator after the execution of the RIM instruction provide this information.Thus, it is essential to look into the Accumulator contents after the RIM instruction is executed. |
| 5 | SIM instruction can be used for serial output of data. | RIM instruction can be used for serial input of data. |
| 6 | Its opcode(in Hex) is 30. | Its opcode(in Hex) is 20. |